Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#03)

Apply Behavioral-level Verilog to Design and

Simulate Combinational Modules

Student Name: Fatima Tariq Roll No: 24i-6557 Section: CE-A

Date performed: 4, August, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1: Write Verilog code for a 4-bit adder/subtractor. The circuit should perform addition or

subtraction depending on the control signal. Simulate the results using a testbench.

Description:

In this task a 4-bit adder and subtractor was implemented in Modelsim using Verilog Language. Registers were used for taking 4 bit inputs and storing 4 bit results.

Code:

module task01(a,b,M,sum,carry); input [3:0] a; input [3:0] b; input M; output reg [3:0] sum; output reg carry; always@(*) begin if (M == 0) {carry,sum}=a+b; else {carry,sum}=a-b; end endmodule module testbench_addsub(); reg [3:0] a; reg [3:0] b; reg M; wire [3:0] sum; wire carry; task01 uut(a,b,M,sum,carry); initial begin a=4'b0001; b=4'b1000; M=1'b1; #50; a=4'b1001; b=4'b1010; M=1'b1; #50; a=4'b0000; b=4'b1110; M=1'b0; #50; a=4'b1011; b=4'b0111; M=1'b0; #50; end endmodule

Output:

Task 2: Write Verilog code for 2x1 MUX having below mentioned operations. Simulate your

program through testbench.

Description:

In this task a 2x1 mux was implemented in Model-sim where if select pin is 0 the AND operation was performed between the inputs and if select pin was set to 1 then OR operation was performed. Desired results were achieved.

Code:

module task02(a,b,s,y); input a,b,s; output reg y; always @(*) begin if(s==0) y=a&b; else y=a¦b; end endmodule module testbench2x1(); reg a; reg b; reg s; wire y; task02 uut(a,b,s,y); initial begin a=1'b0; b=1'b0; s=1'b0; #50; a=1'b0; b=1'b0; s=1'b1; #50; a=1'b0; b=1'b1; s=1'b0; #50; a=1'b0; b=1'b1; s=1'b1; #50; a=1'b1; b=1'b0; s=1'b0; #50; a=1'b1; b=1'b0; s=1'b1; #50; a=1'b1; b=1'b1; s=1'b0; #50; a=1'b1; b=1'b1; s=1'b1; #50; end endmodule

Output:

Task 3: Write Verilog code for a 2-bit comparator that compares two 2-bit binary numbers and

outputs the result as equality, greater than, or less than. Simulate the circuit with a testbench.

Description:

In this task a 2-bit comparator was implemented in Model-sim using Verilog language. And based on the inputs it compared whether the inputs were greater, lesser or equal to each other and produced desired results.

Code:

module task03(a,b,e,g,l); input [1:0] a; input [1:0] b; output reg e; output reg g; output reg l; always @(*) begin if(a==b) begin e=1'b1; g=1'b0; l=1'b0; end else if(a>b) begin e=1'b0; g=1'b1; l=1'b0; end else begin e=1'b0; g=1'b0; l=1'b1; end end endmodule module testbench_comp(); reg [1:0] a; reg [1:0] b; wire e; wire g; wire l; task03 uut(a,b,e,g,l); initial begin a=2'b00; b=2'b00; #50; a=2'b00; b=2'b01; #50; a=2'b00; b=2'b10; #50; a=2'b00; b=2'b11; #50; a=2'b01; b=2'b00; #50; a=2'b01; b=2'b01; #50; a=2'b01; b=2'b10; #50; a=2'b01; b=2'b11; #50; a=2'b10; b=2'b00; #50; a=2'b10; b=2'b01; #50; a=2'b10; b=2'b10; #50; a=2'b10; b=2'b11; #50; a=2'b11; b=2'b00; #50; a=2'b11; b=2'b01; #50; a=2'b11; b=2'b10; #50; a=2'b11; b=2'b11; #50; end endmodule

Output: